SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER ARCHITECTURE COURSE CODE: CSA12

COURSE OUTCOME COVERED

CO1: Analyze the functional units of a computer system including CPU, memory, bus

structures, and I/O components by examining instruction execution cycles, addressing

modes, and performance metrics such as CPI, clock cycles, and benchmarking (BL4)

Assessment Tool Weightage: 40%

QUESTIONS:5 Total Marks:25

ANNEXURE A

APPLICATION BASED QUESTIONS

Code of Conduct:

I _L.VAHINI REDDY_ (Name / Reg No) certify that this submission is my original

work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary

action.

Signature of the student:

1. A smart city surveillance system processes continuous video feeds from multiple cameras, where large volumes of data are transferred between I/O devices, memory, and CPU. During peak hours, the system experiences lag and delayed response in threat detection. Analyze how the interaction between CPU, memory, and I/O contributes to this issue, and explain how different bus structures (single-bus vs. multi-bus) and improvements in instruction execution cycle can enhance system performance.

Answer:

- The CPU experiences delays when memory and I/O devices cannot supply data quickly, creating bottlenecks during peak traffic.

- In a single-bus architecture, CPU, memory, and I/O share one communication path, causing bus contention. A multi-bus architecture allows simultaneous data transfers, improving system performance.

- Techniques such as pipelining, cache memory, and faster instruction execution reduce delays and improve real-time threat detection.

2. A real-time stock trading application running on a processor executes millions of instructions per second, but users report delays in transaction processing during high load. The system has a high CPI due to frequent memory access and branch instructions. Examine how the fetch–decode–execute cycle impacts execution time in this scenario, and propose methods to reduce CPI and improve overall CPU performance using suitable architectural enhancements.

Answer:

- A high CPI increases execution time because memory access delays and branch instructions interrupt the fetch–decode–execute cycle.

- Cache memory, branch prediction, instruction pipelining, and superscalar execution reduce CPU stalls and lower CPI.

- These enhancements improve processor throughput, resulting in faster and more efficient transaction processing.

3. An embedded system based on an 8085 microprocessor is used in an industrial temperature control unit, where sensors provide input and actuators respond accordingly. However, incorrect temperature readings are occasionally processed due to improper use of addressing modes in the program. Discuss how different addressing modes influence data access, identify possible causes of such errors, and suggest how the instruction set of 8085 can be effectively used to ensure accurate and reliable operation.

Answer:

- Addressing modes determine how data is accessed from registers or memory. Incorrect addressing may read the wrong sensor value.

- Errors can occur due to improper use of direct, indirect, or register addressing modes, leading to incorrect temperature processing.

- Using appropriate 8085 instructions such as LDA, MOV, MVI, STA, LXI, IN, and OUT ensures correct data transfer and reliable system operation.

4. A computing system uses an 8086 microprocessor with segmented memory for executing multiple programs simultaneously, but it encounters issues like incorrect data access and stack overflow errors. Evaluate how segmentation (code, data, and stack segments) works in 8086, analyze how improper segment handling can lead to such problems, and explain how instruction execution and addressing modes must be managed to ensure correct program execution.

Answer:

- The 8086 organizes memory into Code Segment (CS), Data Segment (DS), Stack Segment (SS), and Extra Segment (ES) to efficiently manage memory.

- Incorrect segment register values or improper stack handling can result in invalid memory access and stack overflow.

- Proper initialization of segment registers, correct addressing modes, and balanced PUSH and POP instructions ensure accurate program execution.

5. A data communication system represents packet information in hexadecimal format for ease of debugging and transmission, but a software engineer mistakenly interprets values, leading to incorrect data processing. Analyze the importance of number system conversions between hexadecimal and binary in such systems, explain how errors in conversion can affect instruction execution, and discuss how efficient CPU design and benchmarking can help detect and minimize such issues in real-time systems.

Answer:

- Hexadecimal provides a compact and readable representation of binary data, making communication and debugging easier.

- Incorrect hexadecimal-to-binary conversion changes data or instruction values, leading to incorrect program execution.

- Efficient CPU design, benchmarking, and error-checking mechanisms help detect conversion errors and improve the reliability of real-time systems.